Campus core – Cisco Nexus 9K (A48)

- Core switch: sw-aa144-A48 (Cisco Nexus 9K)

  - Mgmt SVI: VLAN 1 – 192.168.2.70/16

  - Acts as campus routing core for the building SVIs below (VOIP and OSPF P2P links).

Allied Health (from SCCC memory)

Building: Allied Health (b-allied-health)

Local / access switches

- sw-ah144 – Dell N2048P stack

  - Role: legacy access, phones + data

  - Mgmt IP: 10.80.40.74

  - Notes: PHY/link instability; several ports drop link or PoE. (Issue issue-dell-phy-failure.)

- SW-AA144-WB2 – Aruba JL679A (WB2 box)

  - Role: replacement access for Allied Health phones

  - Mgmt IP: 192.168.2.215

  - VLANs at edge: data VLAN 1, voice VLAN 322 (from LLDP/Link-Live)

Nexus 9K connections

- In the Nexus config, there is no SVI explicitly named “Allied Health” or “AH144”.

- Allied Health traffic today rides on VLANs 1 / 322 / 840 toward the core, but the 9K only exposes SVIs for the campus VOIP subnets (10.30.x.x) and OSPF P2P / building links (172.20.2.x), not for VLAN 322/840 directly.

- So: Allied Health is logically behind the core, but its exact 9K SVI name is not labeled “Allied Health” in the config.

A144 building

Local / distribution / access switches (from Link-Live)

- SW-A144-1-aruba – Aruba JL659A

  - Mgmt IPs observed: 10.40.2.248, 192.168.2.201

  - VLANs:

    - Data: VLAN 402, sometimes VLAN 1

    - Voice: VVLAN 301 (phones)

- SW-A144-2-Aruba – Aruba JL659A

  - Mgmt IP: 172.20.1.11

  - VLANs: VLAN 610 (data), VVLAN 301 (voice)

- SWA-AA144-Access – Aruba JL659A

  - Mgmt IP: 10.40.14.1

  - VLANs: VLAN 414 (data) plus other tagged VLANs (e.g., 670 toward firewall)

(All three are clearly A144-related by hostname and VLAN usage.)

Nexus 9K connections for A144

From the Nexus SVIs:

- VLAN 301 – 10.30.1.1/24

  - Description: VOIP in A144

  - Function: A144 VOIP subnet (phones).

- VLAN 302 – 10.30.2.1/24

  - Description: VOIP in AA105 & AA144

  - Function: Shared VOIP subnet between AA105 and A144.

- VLAN 622 – 172.20.2.41/30

  - Description: OSPF10-A144-1

- VLAN 623 – 172.20.2.45/30

  - Description: OSPF10-A144-2

- VLAN 614 – 172.20.2.9/30

  - Description: OSPF10-AA144-A48Cisco9k

- VLAN 629 – 172.20.2.70/30

  - Description: OSPF-AA144-Aruba0

Taken together, A144 has:

- One VOIP subnet (301) just for A144

- One shared VOIP subnet (302) with AA105

- Three OSPF point-to-point / routed links (VLAN 614, 622, 623) plus one labeled AA144-Aruba0 (629), likely the routed uplink into the A144 Aruba stack.

AA105 building

Local / distribution / access switches

- SWA-AA105 – Aruba JL659A

  - Mgmt IPs seen: 10.70.34.1, 10.70.37.1, 192.168.2.199

  - VLAN roles observed:

    - VLAN 302 / VVLAN 302 – VOIP phones (AA105 & A144)

    - VLAN 733 / 734 / 737 / 375 – data/camera VLANs (e.g., 10.70.33.1, 10.30.75.1)

Nexus 9K connections for AA105

- VLAN 302 – 10.30.2.1/24

  - Description: VOIP in AA105 & AA144

- VLAN 625 – 172.20.2.53/30

  - Description: OSPF10-AA105

So AA105’s distribution switch (SWA-AA105) connects back to the 9K via an OSPF P2P link in VLAN 625, and its phones reside in VOIP subnet 10.30.2.0/24 (VLAN 302) on the 9K.

H128 building

Local / distribution / access switches

- swa-h128 – Aruba JL659A

  - Mgmt IP: 10.70.89.1

  - VLANs: VLAN 789 (data), VVLAN 304 (voice)

  - Used heavily in 5G PoE tests with LinkRunner; good DNS/gateway performance.

Nexus 9K connections for H128

- VLAN 613 – 172.20.2.5/30

  - Description: OSPF-H128-CiscoA48

So H128 has a routed OSPF P2P link on VLAN 613 back to the 9K, while phones ride voice VLAN 304, which is handled by the Aruba swa-h128 and routed by the core.

B101 building

Local / distribution / access switches

- SW-B101-Aruba – Aruba JL660A

  - Mgmt IP: 10.80.33.1

  - VLANs: VLAN 833 (cameras/data)

Nexus 9K connections for B101

- VLAN 620 – 172.20.2.33/30

  - Description: OSPF10-B101

That gives B101 a dedicated OSPF P2P link from the 9K, presumably terminating on the B101 distribution switch.

SU121 & SA208

Local / access switches

- Individual switch names aren’t explicitly captured in the snippets we’ve looked at, but building names appear in Nexus VOIP config and OSPF.

Nexus 9K connections

- VLAN 303 – 10.30.3.1/24

  - Description: VOIP in SU121 & SA208

  - Shared VOIP subnet for both buildings.

- VLAN 626 – 172.20.2.57/30

  - Description: OSPF-SU121

So:

- SU121: has both a VOIP subnet (303) and an OSPF P2P link (626).

- SA208: shares the VOIP subnet (303), but its routed link name isn’t explicitly broken out (likely part of same dist stack as SU121 or another path not labeled per-building in this config).

Epworth

Switches

- Distribution/access switch names for Epworth aren’t in the Link-Live snippets we’ve seen yet.

Nexus 9K connection

- VLAN 616 – 172.20.2.17/30

  - Description: OSPF10-Epworth-To-Cisco9k-A48

So Epworth has at least one dedicated OSPF P2P link on VLAN 616 to the 9K.

Tech School

Switches

- No explicit Aruba/Dell hostname captured yet in the PDFs we’ve looked at.

Nexus 9K connection

- VLAN 617 – 172.20.2.21/30

  - Description: OSFP10-TechSchool-To-A48Cisco9k

That’s the Tech School routed link from the 9K.

SLC151 Dorms

Switches

- Access/dist switch names aren’t clearly exposed yet in the Link-Live snippets we saw, but the building appears in the Nexus OSPF descriptions.

Nexus 9K connection

- VLAN 618 – 172.20.2.25/30

  - Description: OSFP10-SLC151-Dorms-To-A48Cisco9k

So the dorms have a dedicated OSPF P2P link on VLAN 618.

V201

Switches

- No explicit Aruba switch name visible in the snippets reviewed, but there is a routed link for V201.

Nexus 9K connection

- VLAN 619 – 172.20.2.29/30

  - Description: OSPF10-V201

TA107

Switches

- No explicit downstream switch hostname in the Link-Live snippets we pulled; TA107 is visible from the 9K only via OSPF.

Nexus 9K connection

- VLAN 621 – 172.20.2.37/30

  - Description: OSPF10-TA107

Sharp CC

Switches

- Switch hostname not clearly visible yet in Link-Live snippets, but the building shows up in OSPF description.

Nexus 9K connection

- VLAN 624 – 172.20.2.49/30

  - Description: OSPF10-SharpCC

COS109

Switches

- No explicit Aruba hostname yet from the snippets we scanned.

Nexus 9K connection

- VLAN 627 – 172.20.2.61/30

  - Description: OSPF-COS109

Health Center

Switches

- Not yet identified by hostname in Link-Live snippets, but present in 9K OSPF.

Nexus 9K connection

- VLAN 628 – 172.20.2.65/30

  - Description: OSPF-Healthcenter

“Core/backbone” links (not really buildings)

For completeness:

- VLAN 611 – 172.20.2.1/30 – OSPF-TO-6509-CORESWITCH (legacy core)

- VLAN 612 – 172.20.3.1/30 – OSPF-TO-9300NEXUS-BuilingT (other 9300/Nexus core)