1. Switches we care about for SNMP polling

These are the ones we’ve seen in Telegraf + Link-Live + memory, plus the one that’s currently missing from SNMP polling.

Cisco Nexus cores (already in SNMP block)

- sw-aa144-A48 – Cisco Nexus 9K – mgmt 192.168.2.70

- Likely peers: other cores 192.168.2.71, 192.168.2.72, 192.168.2.73 (already in ping + SNMP blocks)

Aruba access / distribution switches (already mostly in SNMP)

From [[inputs.snmp]] # Aruba 6300 Switches and Link-Live:

- SW-B101-Aruba – Aruba JL660A – mgmt 192.168.2.196 (B101)

- SW-A144-1-aruba – Aruba JL659A – mgmt 192.168.2.201 / 10.40.2.x (A144)

- SW-A144-2-Aruba – Aruba JL659A – mgmt 192.168.2.202 / 172.20.1.11 (A144)

- SWA-AA105 – Aruba JL659A – mgmt 192.168.2.199 / 10.70.34.1 (AA105)

- swa-h128 – Aruba JL659A – mgmt 10.70.89.1 (H128) (not directly in agents list yet, but reachable via routed IP)

- A bunch of other dorm / SA / SU switches already in the agents = [...] list (SLC151, SLG, SLJ, SLS, etc.).

Allied Health (the missing one)

- sw-ah144 – Dell N2048P – mgmt 10.80.40.74 (legacy, unstable)

- SW-AA144-WB2 – Aruba JL679A – mgmt 192.168.2.215 (replacement WB2 switch)

Currently:
SW-AA144-WB2 (192.168.2.215) is not in the SNMP agents list and not in the ping URLs.

We’ll focus the how-to using these three platforms:

- Aruba AOS-CX (6300/6100 family: most access switches, including SW-AA144-WB2)

- Cisco Nexus 9K (core)

- Dell N2048P (legacy Allied Health stack)

2. Step-by-step: configure SNMPv3 on each switch type

2.1 Common design: one campus SNMPv3 user

Your telegraf.conf already uses SNMPv3 with:

- version = 3

- sec_name = "MonitorUser"

- auth_protocol = "SHA"

- priv_protocol = "AES"

We’ll keep that design:

Goal:
Create an SNMPv3 user named MonitorUser with authPriv (SHA + AES) on every switch, then just add the switch IP to telegraf.conf.

Below, I’ll call the real auth/priv passwords <AUTH_PASS> and <PRIV_PASS> – keep using your existing secrets; don’t change them unless you intend to rotate.

2.2 Aruba AOS-CX (6300/6100 + WB2)

Applies to:
SW-AA144-WB2, SWA-AA105, SW-A144-1-aruba, SW-A144-2-Aruba, SW-B101-Aruba, swa-h128, dorm switches, etc.

On the switch CLI:

- Enter config mode

- configure terminal

- Create an SNMPv3 group

- snmpv3 group MonitorGroup auth-priv

- Create the SNMPv3 user

- snmpv3 user MonitorUser group MonitorGroup auth sha <AUTH_PASS> priv aes <PRIV_PASS>

- (Optional) Set a friendly syslocation / contact

- snmp-server location "Allied Health - Main Closet"

- snmp-server contact "SCCC Network Team"

- Enable SNMP agent (usually on by default)

- snmp-server enable

- Save the config

- write memory

- Test from the Telegraf host

From the Telegraf server:

snmpwalk -v3 -l authPriv -u MonitorUser -a SHA -A '<AUTH_PASS>' -x AES -X '<PRIV_PASS>' 192.168.2.215 1.3.6.1.2.1.1.5.0

You should see the switch’s sysName come back.

Repeat the same steps on other Aruba switches where you want SNMPv3 polling (if they’re not already configured).

2.3 Cisco Nexus 9K

You already have a working SNMPv3 setup (Telegraf is polling the 9K block now), but here’s the pattern if you bring up another core or need to re-do it.

On the Nexus:

- Enter config mode

- configure terminal

- Create a group

- snmp-server group MonitorGroup v3 priv

- Create the user

- snmp-server user MonitorUser MonitorGroup v3 auth sha <AUTH_PASS> priv aes-128 <PRIV_PASS>

- (Optional) Set location/contact

- snmp-server location A144 Core Closet

- snmp-server contact "SCCC Network Team"

- Confirm

- show snmp user

- show snmp group

- Test from Telegraf host

- snmpwalk -v3 -l authPriv -u MonitorUser -a SHA -A '<AUTH_PASS>' -x AES -X '<PRIV_PASS>' 192.168.2.70 1.3.6.1.2.1.1.5.0

2.4 Dell N2048P (legacy Allied Health stack)

Dell N-Series syntax varies slightly by firmware, but typical CLI looks like:

- Enter config mode

- configure

- Create a group

- snmp-server group MonitorGroup v3 priv read default

- Create the user

- snmp-server user MonitorUser MonitorGroup v3 auth sha <AUTH_PASS> priv aes <PRIV_PASS>

- Enable SNMP

- snmp-server enable

- Set location (optional)

- snmp-server location "Allied Health - Dell stack"

- Save

- copy running-config startup-config

- Test from Telegraf host

- snmpwalk -v3 -l authPriv -u MonitorUser -a SHA -A '<AUTH_PASS>' -x AES -X '<PRIV_PASS>' 10.80.40.74 1.3.6.1.2.1.1.5.0

If Dell gives you any syntax error, use its built-in ? help, but the idea is the same: v3, authPriv, SHA + AES, user = MonitorUser.

3. Update telegraf.conf for SNMP polling

Now that the switches are ready, we just plug them into your existing SNMP inputs.

3.1 Back up the config

On the Telegraf server:

sudo cp /etc/telegraf/telegraf.conf /etc/telegraf/telegraf.conf.bak.$(date +%F)

3.2 Add Allied Health WB2 to ping (optional but recommended)

In the existing [[inputs.ping]] block (building connectivity), you already ping a list of cores + building switches.

Add "192.168.2.215" to that urls array with a comment:

[[inputs.ping]]

urls = [

# Core switches

"192.168.2.70",  # sw-aa144-A48

"192.168.2.71",  # sw-aa144-A24

"192.168.2.72",  # Additional core

"192.168.2.73",  # Additional core

# Administrative Buildings

"192.168.2.199",  # SWA-AA105

"192.168.2.201",  # SW-A144-1-aruba

"192.168.2.202",  # SW-A144-2-Aruba

"192.168.2.215",  # SW-AA144-WB2 (Allied Health WB2)

...

]

3.3 Add Allied Health WB2 to Aruba SNMP block

Your Aruba 6300 SNMP block already polls a long list of 192.168.2.x + the routed IPs.

We just extend that list:

###############################################################################

# SNMP – Aruba 6300 Switches (numeric OIDs only)

###############################################################################

[[inputs.snmp]]

interval = "60s"

timeout  = "5s"

retries  = 2

agents = [

"udp://192.168.2.196:161", "udp://192.168.2.201:161", "udp://192.168.2.1:161",

"udp://192.168.2.40:161",  "udp://192.168.2.171:161", "udp://192.168.2.175:161",

"udp://192.168.2.179:161", "udp://192.168.2.181:161", "udp://192.168.2.182:161",

"udp://192.168.2.183:161", "udp://192.168.2.184:161", "udp://192.168.2.186:161",

"udp://192.168.2.187:161", "udp://192.168.2.188:161", "udp://192.168.2.189:161",

"udp://192.168.2.190:161", "udp://192.168.2.195:161", "udp://192.168.2.199:161",

"udp://192.168.2.200:161", "udp://192.168.2.202:161",

"udp://192.168.2.203:161", "udp://192.168.2.204:161", "udp://192.168.2.205:161",

"udp://192.168.2.212:161", "udp://10.70.1.1:161",    "udp://10.70.34.1:161",

"udp://192.168.2.215:161"  # SW-AA144-WB2 (Allied Health WB2) <-- NEW

]

version       = 3

sec_name      = "MonitorUser"

auth_protocol = "SHA"

auth_password = "<AUTH_PASS>"   # keep your existing value

priv_protocol = "AES"

priv_password = "<PRIV_PASS>"   # keep your existing value

sec_level     = "authPriv"

# (rest of OID fields/tables unchanged)

Minimal change: we only added one line at the bottom of the agents array.

3.4 (Optional) Add Dell sw-ah144 to SNMP

If you want to monitor the legacy Dell stack 10.80.40.74 via SNMPv3 too:

- Make sure SNMPv3 MonitorUser is configured on the Dell (see 2.4).

- Then add to the same agents list or create a small “Dell SNMP” block. Using the same block is easiest:

...

"udp://192.168.2.215:161",  # SW-AA144-WB2 (Allied Health WB2)

"udp://10.80.40.74:161"     # sw-ah144 (Dell N2048P Allied Health) <-- OPTIONAL

If Dell can’t do v3 sanely, you could instead add a separate [[inputs.snmp]] block using version = 2 and a read-only community; just keep that community name out of Git/Chat.

3.5 (Optional) Add missing Nexus 9K core to SNMP

Your Nexus block currently polls 192.168.2.70, 2.72, 2.73, but pings also include 2.71.

If 192.168.2.71 is also a 9K or core you care about, add it here:

###############################################################################

# SNMP – Cisco Nexus 9000 (numeric OIDs only)

###############################################################################

[[inputs.snmp]]

interval        = "60s"

timeout         = "5s"

retries         = 2

max_repetitions = 10

agent_host_tag  = "source"

agents = [

"udp://192.168.2.70:161",

"udp://192.168.2.71:161",  # NEW if this is a 9K core

"udp://192.168.2.72:161",

"udp://192.168.2.73:161"

]

version       = 3

sec_name      = "MonitorUser"

auth_protocol = "SHA"

auth_password = "<AUTH_PASS>"

priv_protocol = "AES"

priv_password = "<PRIV_PASS>"

sec_level     = "authPriv"

# (rest of OIDs unchanged)

3.6 Restart Telegraf and verify

After editing:

sudo telegraf --config /etc/telegraf/telegraf.conf --test | less   # quick sanity check

sudo systemctl restart telegraf

sudo systemctl status telegraf

Then in Grafana / Influx:

- Filter by source = "192.168.2.215" and confirm you see interface / interface_legacy metrics arriving.

- Do the same for 10.80.40.74 if you enabled Dell SNMP.

4. One security reminder

Your current telegraf.conf includes:

- An InfluxDB token

- SNMPv3 auth/priv passwords for MonitorUser

I’ve redacted them here on purpose. Since the file’s been shared around a bit, it’s a good idea to:

- Plan a token rotation for InfluxDB.

- At some point, rotate MonitorUser passwords on all devices and update Telegraf to match.