# Telegraf SNMP and NetFlow Configuration – Aruba & Fortinet

This guide provides step-by-step instructions for configuring Telegraf SNMP inputs for Aruba CX switches and Fortinet firewalls, as well as the NetFlow listener configuration for Fortinet traffic analytics.

# 1. Telegraf SNMP Input – Aruba CX Switches

File: telegraf.d/aruba_snmp.conf

[[inputs.snmp]]
  agents = ["<SWITCH_IP>"]
  version = 3
  sec_name = "MonitorUser"
  auth_protocol = "SHA"
  auth_password = "StrongAuthPass"
  priv_protocol = "AES"
  priv_password = "StrongPrivPass"
  sec_level = "authPriv"
  timeout = "5s"
  retries = 3
  name = "if_stats"
  interval = "60s"

  [[inputs.snmp.field]]
    name = "hostname"
    oid = "SNMPv2-MIB::sysName.0"

  [[inputs.snmp.table]]
    name = "interface"
    oid = "IF-MIB::ifTable"
    inherit_tags = ["hostname"]

  [[inputs.snmp.table.field]]
    name = "ifDescr"
    oid = "IF-MIB::ifDescr"

  [[inputs.snmp.table.field]]
    name = "ifHCInOctets"
    oid = "IF-MIB::ifHCInOctets"

  [[inputs.snmp.table.field]]
    name = "ifHCOutOctets"
    oid = "IF-MIB::ifHCOutOctets"

  [[inputs.snmp.table.field]]
    name = "ifOperStatus"
    oid = "IF-MIB::ifOperStatus"

  [[inputs.snmp.table.field]]
    name = "ifSpeed"
    oid = "IF-MIB::ifSpeed"

  [[inputs.snmp.table.field]]
    name = "ifHCInErrors"
    oid = "IF-MIB::ifHCInErrors"

  [[inputs.snmp.table.field]]
    name = "ifHCOutErrors"
    oid = "IF-MIB::ifHCOutErrors"

  [[inputs.snmp.table.field]]
    name = "ifInDiscards"
    oid = "IF-MIB::ifInDiscards"

  [[inputs.snmp.table.field]]
    name = "ifOutDiscards"
    oid = "IF-MIB::ifOutDiscards"

  [inputs.snmp.tags]
    device_role = "access"
    building = "Library"
    vendor = "Aruba"

Replace <SWITCH_IP> with the Aruba switch's management IP. Duplicate this block for each switch.

# 2. Telegraf SNMP Input – Fortinet Firewall

File: telegraf.d/fortinet_snmp.conf

[[inputs.snmp]]
  agents = ["<FORTINET_IP>"]
  version = 3
  sec_name = "MonitorUser"
  auth_protocol = "SHA"
  auth_password = "StrongAuthPass"
  priv_protocol = "AES"
  priv_password = "StrongPrivPass"
  sec_level = "authPriv"
  timeout = "5s"
  retries = 3
  name = "fortinet_stats"
  interval = "60s"

  [[inputs.snmp.field]]
    name = "hostname"
    oid = "SNMPv2-MIB::sysName.0"

  [[inputs.snmp.field]]
    name = "cpu"
    oid = "FORTINET-FORTIGATE-MIB::fgSysCpuUsage.0"

  [[inputs.snmp.field]]
    name = "mem"
    oid = "FORTINET-FORTIGATE-MIB::fgSysMemUsage.0"

  [[inputs.snmp.field]]
    name = "sessions"
    oid = "FORTINET-FORTIGATE-MIB::fgSysSesCount.0"

  [inputs.snmp.tags]
    device_role = "firewall"
    building = "Datacenter"
    vendor = "Fortinet"

Replace <FORTINET_IP> with the FortiGate's internal or management IP.

# 3. Telegraf NetFlow Input – Fortinet

File: telegraf.d/netflow_listener.conf

[[inputs.netflow]]
  service_address = ":2055"
  max_udp_packet_size = 65535
  protocols = ["netflow5", "netflow9", "ipfix"]
  tags = { source = "fortinet" }
  log_packet_metadata = false

This listens on UDP port 2055 to ingest flows exported by the Fortinet firewall.

# 4. Setup Instructions

Step 1: Place Configuration Files

Save each configuration snippet to the following paths on the Telegraf server:

/etc/telegraf/telegraf.d/aruba_snmp.conf

/etc/telegraf/telegraf.d/fortinet_snmp.conf

/etc/telegraf/telegraf.d/netflow_listener.conf

Step 2: Test SNMP Access (optional)

Run this command from the Telegraf host to verify SNMPv3 access to a device:

snmpwalk -v3 -l authPriv -u MonitorUser -a SHA -A StrongAuthPass -x AES -X StrongPrivPass <device_ip> sysDescr

Step 3: Restart Telegraf

If using systemd:

sudo systemctl restart telegraf

If using Docker:

docker restart telegraf

Step 4: Validate Logs

For systemd deployments:

journalctl -u telegraf -f

For Docker deployments:

docker logs -f telegraf

Ensure that SNMP data and NetFlow packets are being received successfully.